# STAGE 4 – ACCESSORIES

# A1.2.000.3C6.4.XXX

Version 4.0 – January 2023

| Description: |  |
| --- | --- |
| Starter Lead for Power Circuits |  |
| 3 Core x 4.0mm² |  |
| Low Fire Hazard (XLPE) Cable |  |
| Cable Colour: | Grey |

Please read instructions before commencing installation and retain for future reference.

Technical Specification:

| Material: | Connector: |
| --- | --- |
|  | PA66 + 25% GF UL-94 V0 (S/N 704872-1) |
|  | PA66 unfilled UL-94 V2 (S/N 1573372-1) |
| Connector Dimensions: | 39.3mm (L) x ø 19mm (W) |
| Cable Insulation: | HFS-90-TP Sheath (GY) |
| Cable Rating: | 450/750V to AS/NZS 5000.2 |
| Minimum Bending Radius: | Fixed: 70mm |
|  | During Installation: 100mm |
| Overall Cable Diameter: | 11.5mm |
| Maximum Operating Temperature: | 90°C |
| Enclosure Class: | IP20 |
| Rated Current (A): | 25A |
| Power Outputs: | 4 Pole (3 Wire) |
| Rated Voltage (V): | 240V |
| Frequency (Hz): | 50Hz |
| Wire Size (mm²) | 4.0mm² |
| Weight: | 236g (based on 1m) |
| Wiring & Test Standard: | AS/NZS 3000 |
| Connector Coupler Standard: | AS/NZS 61535:2011 |